## Overview

- 2.4G 4x4 Wi-Fi (802.11ax ) 20/40MHz channel width

- 5G 4x4 Wi-Fi (802.11ax) 20/40/80/160MHz channel width

- 2 x 2.4G/5G dual-band antennas

- 1 x 1GE LAN

- 1 x Reset button

- Generic Ceiling Mounting

Manufacturer: SonicFi

## Specifications

Radios:

- Concurrent Dual Band 2.4 GHz, 5 GHz

- 802.11ax 2x2 MU-MIMO supporting up to 3Gbps data rate

Interfaces:

- Uplink: Ethernet Port(RJ45)*1

- PoE-PD: 802.3at

Physical

- Dimension (Diameter x Depth): 154 mm x 23.6 mm

- Input Voltage

  - PoE input: 44 ~ 57V DC, 802.3at PoE (PD)

- Weight: 0.28 Kg

- 1 X Di-Color LED (W/B)

- Buttons: Factory reset

- Environmental Conditions

  - Operating Temperature: 32°F (0°C) to 104°F (40°C)

  - Operating Humidity: 10% to 90% non-condensing

- Power Consumption: 13.5 W max

- Antenna Pattern (build-in Antennas)

  - 2.4GHz – 4.0 (dBi) max

  - 5GHz - 5.0 (dBi) max

- Mounting options

  - Ceiling mount

Radio

- Standards 802.11ax (Wi-Fi 6) concurrent dual-band 2.4 & 5 GHz

- Supported Data Rates

  - 802.11b: 1, 2, 5.5, 11 Mbps

  - 802.11a/g: 6, 9, 12, 18, 24, 36, 48, 54 Mbps

  - 802.11n: 6.5-300 Mbps(20 / 40 MHz)

  - 802.11ac: 6.5-867 Mbps(20 / 40 / 80 MHz)

  - 802.11ax: 3.6-574 Mbps(2.4 GHz, 20 / 40 MHz)

  - 802.11ax: 3.6-2402 Mbps(5 GHz, 20/ 40 / 80 / 160 MHz )

- Channelization

  - 2.4 GHz: 20 / 40 MHz

  - 5 GHz: 20 / 40 / 80/160MHz

- Frequency Range

  - 2400-2484GHz

  - 5150-5250MHz

  - 5250-5350MHz

  - 5470-5725MHz

  - 5725-5850MHz

- Certifications

Regulatory Compliance

- Safety

  - NRTL Listed 62368-1 (US & CA)

  - CB with UL/IEC/EN 62368-1 (Basic safety certificate for worldwide marketing)

  - GB 9254 -2008(Class B of Product)

  - EN55032, CISPR 32:2010

  - EN55024, CISPR 24:2010

  - CCC (optional)

  - SRRC (optional)

  - FE (optional)

  - FCC (optional)

  - RoHS 2011/65/EU compliant (RoHS 11 compliant, no Pb)

  - WEEE 2002/96/EC recyclable materials requirements

## What’s in the box

Package Content:

- RAP630C-311G Indoor AP access

- Ceiling Mount Kit

- Quick Installation Guide

## Interface overview

- System LEDs

- Factory reset button

- 1 x 1G Ethernet Port

- 10/100/1000 Mbps Ethernet Port (802.3at/af POE input)

- Auto-sensing linking speed(10/100/1000BASE-T)

- PoE LED and Mode button

## System LEDs

- Flashing White / Off every 0.5 sec: The device is initialing and booting up

- Steady White: The device is awaiting adoption

- Steady Blue: The device is adopted and is in normal operating mode

- Quickly Flashing White / Blue: The device is currently being upgraded - don't interrupt the process

- Blue and Flashing off every 5 sec: Access point has lost network connectivity and is searching for wireless link