## Overview

- WiFi: 6 Dual Band 2x2 (2.4/5GHz)

- Uplink: 1 x 2.5Gb (PoE in)

- Pole mount

- IP67

Manufacturer: Actiontec

Country Support: US, Canada

## Specifications

Radios:

- Concurrent Dual-Band 2.4 GHz & 5 GHz

- 802.11ax 2x2 MU-MIMO supporting up to 1.774 Gbps data rate

- Bluetooth Low Energy (BLE) 5.0

Interfaces:

- Uplink: 1 x 2.5Gb RJ-45, PoE in 802.3at PoE (PD)

Physical

- Dimension (W x D x H): 260 mm (W) x 140 mm (D) x 71 mm (H)

- Input Voltage

  - 50V ~ 57V, 802.3 at PoE (PD)

- Weight: 1.5 Kg (without bracket)

- 1 X Tri-Color LED (R/G/B)

- Buttons: Restart

- Environmental Conditions

  - Operating Temperature: -40°F (-40°C) to 149°F (65°C)

  - Operating Humidity: 5% to 95% non-condensing

  - Elevations: 86kPa～106kPa altitude

  - Dustproof and Waterproof: IP67

- Power Consumption: < 20 W

- Antenna Pattern (built-in Antennas)

  - Polarization: Horizontal/Vertical

  - Peak Gain: 5/7 dBi

  - Efficiency (Avg): >70%(2400-2500MHz)

  - Efficiency (Avg): >75%(4940-5850NHz)

  - Return Loss: 10dB

  - Isolation: 20dB

  - H-Beam Width: 120 degree

  - V-Beam Width: 60 degree

- Mounting options

  - Wall mount

  - Pole mount

Radio

- Standards 802.11ax (Wi-Fi 6) concurrent dual-band 2.4 & 5 GHz

- Supported Data Rates

  - 802.11b: 1, 2, 5.5, 11 Mbps

  - 802.11a/g: 6, 9, 12, 18, 24, 36, 48, 54 Mbps

  - 802.11n: MCS0~MCS7

  - 802.11ac: MCS0~MCS9

  - 802.11ax: MCS0~MCS11

- Channelization

  - 2.4 GHz: 20 / 40 MHz

  - 5 GHz: 20 / 40 / 80MHz

- Frequency Range

  - 2.4000GHz~2.4835GHz

  - 5.150~5.250,5.250~5.350,5.470~5.725, 5.725~5.850 GHz

- Certifications

Regulatory Compliance

- Safety

  - NRTL Listed 62368-1 (US & CA)

  - CB with IEC/EN 62368-1 (Basic safety certificate for worldwide marketing)

  - GB 9254 -2008(Class B of Product)

  - EN55032, CISPR 32:2010,EN55024, CISPR 24:2010

  - Level 4; EN61000-4-2, FCC Part 15B

  - WEEE 2002/96/EC recyclable materials requirements