## Overview

- Indoor Dual Band Dual Concurrent 2+2 802.11BE Access Point

- Supports 2.4GHz 2x2 and 5GHz 2x2 radio circuits

- Achieves up to 5Gbps total wireless throughput

- Equipped with rich flash and SDRAM for dual image, secured boot, and advanced system features

- Ideal for MDUs, hotels, and enterprise deployments

- Includes BLE and Zigbee support for IoT use cases (*SKU-dependent)

- Sleek, compact design that mounts cleanly to wall plates or flat surfaces

Manufacturer: SonicFi

## Specifications

Interface

- WAN Port:

  - 1 × RJ45 Ethernet port

  - Auto-sensing: 10/100/1000/2500 BASE-T

  - Power Input: PoE-PD 802.3at

- LAN Ports:

  - 3 × RJ45 Ethernet ports

  - Auto-sensing: 10/100/1000/2500 BASE-T

  - PoE-PSE: 802.3at

Wi-Fi Radios

- 2.4 GHz Radio

  - Standard: 802.11BE 2x2 (2SS)

  - Max Data Rate: 688 Mbps

- 5 GHz Radio

  - Standard: 802.11BE 2x2 (2SS)

  - Max Data Rate: 4323 Mbps

- Architecture: Dual Band MU-MIMO

Wireless Antenna

- Type: 3 integrated antennas for 2.4G, 5G, and BLE

- Antenna Gain:

  - 2.4GHz: 4 dBi

  - 5GHz: 5 dBi

  - BLE: 1.78 dBi

Physical

- Dimensions: Compact, vertically mounted housing

- Weight:

  - Net: 330g

  - With bracket: 400g

- Mounting:

  - Over-the-socket wall plate mount

  - Generic wall surface mount (kit included)

Environmental Conditions

- Operating Temperature: 0°C to 40°C / 32°F to 104°F

- Storage Temperature: –40°C to +70°C / –40°F to +158°F

- Operating Humidity: 10% to 90% (non-condensing)

- Storage Humidity: 5% to 90% (non-condensing)

Key Features

- High-efficiency Wi-Fi 7 (802.11BE) with MLO and MRU

- All-in-one network and IoT access point with optional BLE and Zigbee

- Dual concurrent radios for high user density performance

- PoE-powered with PoE pass-through LAN port for powering downstream devices

- Toolless installation with universal wall mount options

## What’s in the box

Package Content:

- RAP750W-311A Access Point

- Mounting Brackets (Over-the-Socket + Generic)

- Screw and anchor set

- Quick Start Guide

- Regulatory & Compliance Documentation